| URS for 30kL Fermenter |  |
| --- | --- |
| Equipment | Fermenter 3000L |
| URS number | XX/X/URS/FER-019 |
| Version No. | 01 |
| Effective Date | 10/12/2015 |

This document and its contents are the confidential information of ABC Private Limited. No    part of this document may be copied, reproduced, modified, or distributed to any third party, in any form or by any means, or stored in a database or retrieval system, without prior written permission of ABC Private Limited.

Table of Contents

1.	Overview and Description	3

2.	Facility Details	6

3.	Technical Data and System Specification  	6

4.	Automation, Data and Security	12

5.          Utility Requirements	14

6.	Other Requirements	14

6.6.1	General Guidelines	15

6.6.2	Sight and Light glass	16

6.6.3	Bottom connections	16

6.6.4	Surface finish of the vessel	16

6.6.5	Vessel Insulation	16

7.		18

8.	 Revision History	18

9. Approval Details ………………………………………………………………………………………………19

# Overview and Description

This document defines only the user requirements for the Production fermenters and its Utility, Ancillary equipment. This document encompasses the normal range of the Fermenters and its ancillary equipment operations.

The purpose of the Fermenter System is to perform cell growth in a controlled environment. Media to the fermenter can be supplied from any of the Media Preparation Vessels through (MPV) transfer panels or shall be prepared in the same tank itself.  The pH of the media is adjusted as required & is sterilized in the fermenter at 122°C for 30 to 60 Min/ for required duration. Cells are inoculated into the fermenter via using air pressure and are then cultured in the fermenter system under closed conditions. For cell growth, the aeration system design would be able to deliver the sterile air to maintain the oxygen requirement during the growth and If in case of anaerobic respiration Nitrogen gas sparging system to be provided.

Dissolved Oxygen concentration can be maintained by cascade control via Air flow, Oxygen flow, Agitation, and Backpressure Controlling system.

Sterilized Acid, Caustic, Antifoam, and nutrients are supplied from Dosing Vessels or any containers, using air pressure or peristaltic pumps for the microbial growth as required.

The fermentation process would be a fed batch or continuous where the flow of substrates/nutrients will be controlled through control valve. The fermenter shall be fed with the various nutrients required for the process intermittently and some of them will be fed continuously.

There shall be Partial Harvesting of the broth collected aseptically via dip tube to harvest tanks in order to maintain the broth level in the vessel comfortably. Samples of the Fermenter will be withdrawn aseptically and are analyzed in the Quality lab for cell density, pH, and viability. Once the required Cell mass reached, the broth will be transferred from Fermenter to the harvest tank or inter transfer to other fermenters & taken forward to the purification suite as required.

Fermenter working volume in ungassed condition should be 30000 liters vendor has to calculate the gross volume considering the agitation & the aeration of 1.5 VVM.

The equipment shall be capable of producing Microbial cells like, Fungal & Yeast fermentation product continuously and the Fermenters is to be located in CNC area with hazard area classification as ‘safe’ zone.

- Media Charging and Transfers:

- There will be media preparation which will happen in MPV and then same shall be transfer to Fermenter or Median shall be prepared in the same fermenter itself and Nutrients from nutrients dosing vessel (NDV) or from any sterile containers at various phases of fermentation. There are various nutrient dosing vessels (NDV) will be catering to the fermenters and after fermentation ends, same will be transferred to purification stages.

- SIP:

For steam sterilization of media, the initial heating of the fermenter shall be done through jacket till 90 to 100°C. And thereafter shall be direct steam injection into the fermenter from the Sparger and dip tubes till the vessel temperature reaches 122 ±1°C. There shall also be provision for steam sterilizing the entire supporting process piping for fermenter which are connected to the vessel. Lines that are steam sterilization in-place should be sloped to facilitate gravity drainage of condensate. All the probes and sensors attached to the fermenter shall be steam Sterilizable.

Automatic sequences for the SIP functions should be programmed into the automation system. As a result all valve systems relevant to sterilization procedures are automated. Steam traps should be equipped with temperature sensors (wherever required) to ensure that the sterilization temperature has been reached. Individual steam traps to be provided for each stage in the jacket with strainer, vacuum breaker and non- return valve. There shall also be steam cross in the transfer line to isolate sterile and non-sterile areas.

- Sparger:

Sparger line for air supply for the equipment and the design should be with better to maintain positive pressure during the fermentation, and the design of the Sparger should be such  that, the media component should not be back flow through Sparger holes.

- Exhaust System:

There shall be exhaust system for venting purpose and also for controlling the back pressure inside the fermenter. The exhaust system shall have a hydrophobic sterile grade filter, control valve and a heat exchanger. There shall be bypass filter line for exhaust filters and also bypass line for control valve. Since, if there would be problem of filters fouling within the batch operation due to aerosol generation, the design of exhaust system shall ensure that filter fouling do not occur.

- Agitation:

Agitation system should be through top driven agitator with double mechanical seal and shall have the arrangement of Thermo-syphon sterile cooling system with air connection for better lubrication. Impellers system shall be designed as removable and the position of impeller shall be adjustable within the shaft length. If fixing bolts of the impellers are required, they shall have no exposed threads.

- Piping:

All the hygienic pipes shall be as per, ASME BPE – American Society of Mechanical Engineers Bioprocessing Equipment, ASME BPE 2009, Table- DT1. Tubes and fittings shall be joined by closed head orbital welding process. Pipes and fittings to be polished as: internal surface shall be Ra (max) of 0.38 micron these specifications are as per ASME BPE 2009. Passivation has to be carried out as per ASTM 380; Drain ability shall be as per ASME BPE 2009. Material of construction shall be SS 316L, mentioned in ASME 2009 Table MOC-1.All utility (Chilled water, cooling water, chilled glycol, Bio-waste, process air, instrument air, potable water and low pressure plant steam) pipes shall be as per ASME B 36.19, ERW type, and material as per ASTM A 312 TP 304. Flanges shall follow material class of ASTM A182 F304 and follow code of ASME B16.5. There can be common strainers at interconnection between MS piping to SS piping.

Note: Vendor has to specify the MOC of the pipes, fittings, gaskets etc. which would be used for Oxygen and Oxygen Sparger.

- Heat Generation:

The heat liberated is approximately 2.5 Kcal/hr/lit of broth. Vendor can calculate the heat transfer rate possible with chilled water as a utility available at 6°C. Vendor to suggest alternate cooling media with inlet temperature, flow rate and pressure, if this is not sufficient. The same will be discussed during technical discussion.

- Sampling:

There shall be arrangement for sampling the broth aseptically, where in there shall be provision to independently steam sterilize the sampling port along with pre-sterilized sampling bottle for sample collection. Sampling would be done at regular interval of time for checking the product quality.

- CIP/:

All fermenters and associated pipe lines shall be cleaned in place using solutions prepared in the individual tanks. Once the fermenters/vessels CIP solution is prepared, recirculation is done using CIP recirculation pump (recirculation pump shall be dedicated to a group of fermenters/vessels). The temperature of the CIP solution during recirculation shall be maintained through heat exchanger on the recirculation pump unit. CIP solution shall be re-circulated at 80-90°C using the above mentioned heat exchanger.

There shall be a conductivity sensor (0-200 micro Siemens/cm) at recirculation pump side in the drain line to determine conductivity of the wash water at the end of CIP cycle, to check the clean-ability of the vessels. The pump shall also be provided with a casing drain to remove all the residual CIP solution held in the pump.

Spray Ball: The spray balls or nozzles orientation and location inside the vessel shall provide effective clean ability at interiors of fermenter walls, impellers and other parts. Spray ball shall be completely drainable and free of dead lags. Spray ball shall have the whole pattern to ensure that complete coverage is attained.

# Facility Details

| Location | Fermentation Block |
| --- | --- |
| Area | 3000mm x 1428 mm (Approximately) |
| Area Classification | CNC (Hazard area classification as safe) |

# Technical Data and System Specification

- Abbreviations

| URS | User Requirement Specification |
| --- | --- |
| °C | Degree Celsius |
| Min | Minutes |
| VVM | Vessel Volume per Minute |
| CNC | Controlled Not Classified |
| ASME BPE | American Society of Mechanical Engineers Bioprocessing Equipment |
| ASTM | American Society of Testing and Materials. |
| MOC | Material of Construction |
| QA | Quality assurance |
| SIP | Sterilization in place |
| CIP | Clean in place |
| EPDM | Ethylene Propylene Diene Monomer |
| PTFE | Polytetrafluoroethylene |
| DI/DT | Diameter of impeller to Diameter of tank |
| DO | Dissolve Oxygen |
| MPV | Media Preparation vessel |
| NDV | Nutrient Dosing Vessels |
| NA | Not Applicable. |

- Technical Details

| DESCRIPTION | UNIT | DETAILS |
| --- | --- | --- |
| Configuration | NA | Vertical, cylindrical, top and bottom tori spherical welded dish end. |
| Working Volume | L | 30000 |
| Gross Volume | L | * |
| Quantity | No’s | 2 |
| Design Code | NA | ASME SEC VIII DIV.1 |
| L/D ratio | NA | 3:1 |
| Internal Diameter | mm | * |
| Distance b/w TL to TL | mm | * |
| Distance b/w bottom dish to Lower TL | mm | * |
| Distance b/w top dish to top TL | mm | * |
| Distance b/w bottom dish to Floor | mm | * |
| Distance between Top of Agitator and Top dish | mm | * |
| Total Heat Transfer Area | m² | * |
| Heat transfer Area from Bottom Dish | m² | * |
| Heat Transfer Area from Shell | m² | * |
| Heat Transfer Area from Internal coils | m² | * |
|  |  | PROCESS SIDE | UTILITY SIDE |
| Fluid | NA | Fermentation Media | Chilled water / Plant steam |
| MOC | NA | SS 316 L | SS 304 / SS304L |
| Surface Finish for Wetted Parts | NA | Ra ≤ 0.4 µm and electro-polished according to ASME BPE 2009 | * |
| ‘O’ Ring / Packing’s | NA | Ethylene Propylene Diene Monomer EPDM)/ Polytetrafluoroethylene (PTFE) | * |
| Sterilization Temperature | °C | 122 (± 1°C) | 122 (± 1°C) |
| Normal Operating Temperature | 0C | 20 to 40 0C | 10 to 40 0C |
| Maximum Operating Temperature | °C | 130 °C | 130 °C |
| Design Pressure | Bar (g) | * | * |
| Working Pressure | Bar (g) | 2.0 to Full Vacuum | 3.5 |
| Hydrostatic Test Pressure | Bar (g) | 1.5 * Design Pressure | 1.5 * Design Pressure |
| SHELL |  |  |  |
| Material of Construction | NA | SS316L | SS304 |
| Thickness | mm | * | * |
| Joint Efficiency | NA | 1.0 | 1.0 |
| Radiography | NA | 100% | 100% |
| BOTTOM END |  |  |  |
| Material of Construction | NA | (Refer MOC) SS316L | SS304 |
| Thickness | mm | * | * |
| Joint Efficiency | NA | 1.0 | 1.0 |
| TOP END |  |  |  |
| Material of Construction | NA | (Refer MOC) SS316L | SS304 |
| Thickness | mm | * | * |
| Joint Efficiency | NA | 1.0 | 1.0 |
| Radiography | NA | 100% | 100% |
| MOC for nozzles | NA | SS316 L | SS 304 |
| MOC for flanges | NA | SS316 L | SS304 |
| MOC for bolts and nuts | NA | SS316 L | SS304 |
| Gaskets | NA | Silicon | PTFE |
| Sanitary Steam Traps | NA | SS316/SS316L | Utility Each stage should have separate trap to drain condensate during sterilization |

Note:* To be specified by vendor

- Nozzle Orientation Details (Top Dish):

- System Specifications

| NOZZLE No. | DUTY | SIZE OF NOZZLE (mm) | END CONNECTION | MATERIAL | ORIENTATION (degrees) |
| --- | --- | --- | --- | --- | --- |
| * | Manhole/Hand hole | * | Bolted with spring bolts | SS316 L | * |
| * | Agitator Flange | * | * | SS316 L | Cover Center |
| * | 2 No’s Rotary Spray ball/CIP | * | * | SS316 L | * |
| * | Light Glass | * | Bolted with gasket | SS316L With Toughened Glass | * |
| * | Sight Glass | * | Bolted with gasket | SS316L With Toughened Glass | * |
| * | Rupture Disc | * | Sterile Union | SS316L | * |
| * | Air Inlet | * | Sterile Union | SS316L | * |
| * | Spare port | * | Sterile Union | SS316L | * |
| * | Pressure Gauge port | * | Sterile Union | SS316L | * |
| * | Pressure Indicator cum Transmitter | * | Sterile Union | SS316L | * |
| * | Foam Sensor | * | Sterile Union | SS316L | * |
| * | Addition port pH adjustment | * | * Sterile Union | SS316L | * |
| * | Spare with Blind plug | * | * Sterile Union | SS316L | * |
| * | Nutrient transfer | * | Sterile Union | SS316L | * |
| * | Air Exhaust | * | Sterile Union | SS316L | * |
| * | Transfer from Seed Fermenter | * | Sterile Union | SS316L | * |

Note:* To be specified by vendor

- Nozzle Orientation Details (Lateral Ports) :

| NOZZLE No. | DUTY | SIZE OF NOZZLE (mm) | END CONNECTION | MATERIAL | ORIENTATION ( Degrees)/ | LOCATION FROM BOTTOM TL |
| --- | --- | --- | --- | --- | --- | --- |
| * | pH Probe | * | Sterile Union | SS316L | * | * |
| * | Temperature Sensor | * | Sterile Union | SS316L | * | * |
| * | Conductivity Sensor | * | Sterile Union | SS316L | * | * |
| * | Sampling Port | * | Manual Diaphragm Valve | SS316L | * | * |
| * | Spare with Blind plug | * | Sterile Union | SS316L | * | * |

Note:* To be specified by vendor

- Nozzle Orientation details (Bottom Dish)

| NOZZLE No. | DUTY | SIZE OF NOZZLE (MM) | END CONNECTION | MATERIAL | MAKE | LOCATION (degrees) |
| --- | --- | --- | --- | --- | --- | --- |
| * | Port for flush bottom valve | * | * | SS316L | * | Center of Bottom Dish |
| * | Port For DP type pressure transmitter | * | * | SS316L | * | * |

Note:* To be specified by vendor

- Nozzle Orientation Details (Vessel Jacket) :

| NOZZLE No. | DUTY | SIZE OF NOZZLE (MM) | END CONNECTION | MATERIAL | LOCATION (degrees) | LOCATION FROM BOTTOM TL |
| --- | --- | --- | --- | --- | --- | --- |
| * | Jacket Inlet | * | * | SS 304 | * | * |
| * | Jacket Outlet | * | * | SS 304 | * | * |
| * | Spare inlet | * | * | SS 304 | * | * |
| * | Spare outlet | * | * | SS 304 | * | * |

Note:* To be specified by vendor

- Agitation System

| AGITATION COMPONENTS | DESCRIPTION | MOC | REMARKS |
| --- | --- | --- | --- |
| Viscosity of Fermentation Media | Not more than 500cP | NA | * |
| Type of Impeller | Max Flow impellers | SS316L | * |
| Diameter of Impeller | * | SS316L | * |
| DI / DT (Diameter of impeller to Diameter of tank) | * | * | * |
| Number of Impeller | 3 | SS316L | * |
| Agitator | Top Mounted | SS316L | * |
| Seal | Double mechanical seal | * | * |
| Agitator RPM | * | * | * |
| Tip Speed | * | * | * |
| Power Rating of Motor | * | * | * |
| Speed of Motor | * | * | * |
| Power Supply | 440V, 50Hz, 3Phase, AC | * | * |
| Class of Protection | IP 55 | * | * |
| Welded Port for Bearing Assembly | * | SS316L | * |
| Type of Gear Box | N/A | * | * |
| Make of Gear Box | N/A | * | * |
| No of Baffles | * | SS316L | * |
| Width of Baffles | * | * | * |
| Length of Baffles | * | * | * |
| VFD | * | * | Siemens/ Equivalent |
| Shaft Coupling | * | SS316L | * |
| Lantern | * | SS304 | * |

Note:* To be specified by vendor

- Valves

| FUNCTION | DESCRIPTION | TYPE | SURFACE INSIDE | OUTSIDE SURFACE | MAKE |
| --- | --- | --- | --- | --- | --- |
| PROCESS SIDE |  |  |  |  |  |
| Harvest Valve | Aseptic Sterilizable-Forged valves | Flush bottom valve (diaphragm) | SS316L Ra 0.4µm electro polished | * | Novaspetic ITT/ Crane/GEMU/ Equivalent |
| Sampling Valve | Aseptic Sterilizable | Manual | SS316L Ra 0.4µm electro polished | * | Novaspetic/GEMU/ Equivalent |
| Process Side | Aseptic Sterilizable, Forged valves | Diaphragm Valve | SS316L Ra 0.4µm electro polished | * | GEMU/ITT/ Crane/ Equivalent |
| JACKET SIDE |  |  |  |  |  |
| Temperature Circuit | Valves for control of steam and cooling water | Control Valve | * | * | * |
| NA | Pressure safety valve | * | * | * | * |
| NA | Jacket Drain Valve | Ball Valve-Solid ball | * | * | * |
| NA | Vent Valve | * | * | * | * |

Note:* To be specified by vendor

- Heating and Cooling

External coils and internal coils for heat transfer have to be considered. On/ off valves and control valves have to be considered wherever required.

- Other Requirements

| DESCRIPTION | TYPE | MATERIAL | MAKE | Quantity |
| --- | --- | --- | --- | --- |
| Rotary Spray Ball | * | * | * | Min 3 No’s |
| Pre-filter/ Final Filter/ Exhaust Filter | * | * | * | Min 1 No |
| Steam Filter | * | * | * | Min 1 No |

Note:* To be specified by vendor

- Instrumentation and Control

| DESCRIPTION | QUANTITY | PURPOSE | MAKE | MOC |
| --- | --- | --- | --- | --- |
| pH sensor and transmitter | 1 | To measure pH | * | SS316L |
| Temperature Sensor and transmitter | 1 | To measure Temperature | * | SS316L |
| DO2 Sensors | 1 | To measure Dissolved Oxygen | * | SS316L |
| DO cable | 1 | * | * | SS316L |
| Pressure Transmitter | 1 | To measure the Back pressure | * | SS316L |
| DO & pH transmitter | 1 | * | * | SS316L |
| Air Flow meter ( Vortex) | 1 | * | * | SS316L |
| Antifoam Sensor | 1 | * | * | SS316L |
| VFD | 1 | * | * | NA |
| Air Flow control Valve | 1 | * | * | SS316L |
| Back Pressure Control Valve | 1 | * | * | SS316L |
| Air Pressure Regulator | 1 | Reduce Air pressure | * | SS316L |

- Aeration details

The aeration system has to be designed in the flow range of 0-2 VVM. The air flow rate has to be measured by a flow meter. The exhaust air will pass out through a sterile filter. The pre and final filters should be of a standard make including Pall/Millipore/Sartorius.

| DESCRIPTION | QUANTITY | MAKE | MOC |
| --- | --- | --- | --- |
| Diameter of Sparger | 1 | NA | SS316 L |
| Sparger Pipe Size | * | NA | SS 316L |
| Sparger Hole Diameter | * | NA | NA |
| Number of Holes | * | NA | NA |

Note: Air Enrichment with Oxygen need to be discussed

# Automation, Data and Security

The Fermenter should have the minimum of the following automation.

- Software System

The vendor should provide a suitable software (must be 21 CFR part 11 compliance) for controlling Fermenter.

The software system should include the following features:

- Batch data

- Batch initialization

- Data acquisition

- Process parameter controls

- Manual data entry of batch information

- Process parameters trends in graphics

- The operating terminals should only be readable

- The software should run on Windows NT/2000 Microsoft Operating Systems

- Should have Active-X controls

- Should support Object model development

- Should support Open Platform Communication standards

- Should possess Add-on Architecture Technology

- The software should include a full features Data log Editor

- Security Features

- Comprehensive Alarm Editor

- Temperature Control System

The temperature control system will be used for controlling the Fermenter temperature during the Fermentation. The automation should be built in a way as to control both endothermic and exothermic reactions.

- pH Control System

The pH control system will be used to control both acidic and basic pH. The pH probe will 				give feedback to alkali and acid dosing pumps for pH control.

- DO2 Control System

DO2 control system will be used for control of Dissolved oxygen in the Fermentation broth by using the Aeration, Back pressure & agitation system.

- Aeration System

Aeration system will control the Air flow rate into the Fermenter.

- Nutrient Dosing system

Nutrient dosing by manual,

Nutrient option by time programmed

Nutrient option by pH mode (pH stat) and other mode.

- Back Pressure Control System

Back Pressure control system will control the Pressure in the Fermenter using a control valve parameter (0 to 2.bar).

# Utility Requirements.

The following utilities will be made available prior to installment.

| Utility | Requirements |
| --- | --- |
| Steam Cooling water Chilled water Chilled brine Process Air Instrument Air | - To achieve 150°C - 25 to 35°C - 10 to 15°C - -20 to -25°C - 2.8 bar to 3.0 bar - 5.5bar to 6.0 bar |
| . Utility required for the jacket | - Steam, Hot water, Cooling water, Chilled water, Chilled brine. |
| Power available | - 415 V+/-10%, 3 Phase, 50+/- 5% Hz |

# Other Requirements.

- Cleaning Requirements

The equipment should include an integrated CIP system.

- Spray device should effectively clean all the nozzles and projections into the Process vessel.

- MOC of Spray device should be SS 316L.

- Performance of Spray nozzle should not be affected by +20% in flow rate or + 20% in delivered pressure at the designed conditions.

- The Spray device should be easily removable. A positioning device or a mark shall be incorporated into the design to allow for proper location or cleaning of equipment.

- The operating pressure for spray ball should NOT be MORE THAN 1.0 bar.

- The location of the Spray ball within the vessel should be decided by the vendor based on the cleaning capacity.

- Spray ball shall be drainable, shall provide whole pattern to ensure that complete coverage is attained and shall be properly manufactured to minimize corrosion.

- The flow rates required for cleaning and using spray ball should be specified by the vendor.

- The overlay and spray ball should be integrated for effective cleaning.

Note: Automatic sequences for the SIP/CIP functions should be programmed into the DCS/SCADA. As a result all valve systems relevant to CIP and sterilization procedures are automated. SIP protocol for the main vessel should reach a temperature of 121°C for a minimum of 30mins. Steam traps should be equipped with temperature sensors to ensure that the sterilization temperature has been reached.

- Sterilization Requirements

Sterilization shall be performed in all the stages like empty vessel and fully loaded so the system should be applicable for manual mode and auto programmed.

- Preventive maintenance and Breakdown maintenance services

Maintenance activities shall be performed any time, so the equipment should have the flexibility to do all the activities.

- Calibration and Instrument breakdown services

All the activities shall be carried out during the process so the equipment should have the calibrated based the design.

- Safety Requirements

Equipment should be fabricated with respect to all safety norms.

- other detail

### General Guidelines

Following recommendations have to be followed during the Design and Fabrication of the Process vessel.

Internal Horizontal Product contact surfaces shall be minimized.

- Process vessel shall be drainable and free of areas where liquid may be retained.

- Fasteners or threads shall not be exposed to the Process steam or cleaning fluids. Bolted attachments should be eliminated wherever possible.

- No engraving or embossing of materials (for identification or traceability reasons) should be done on the Process contact side. When markings are required on the Process contact surfaces, other methods of identification should be used. All Product contact surfaces shall reach the required temperatures during Steam Sterilization cycles. –Able to Validate-Options to be discussed

- When Nonmetallic materials are used (E.g., Plastics, elastomers, or adhesives), they have to comply FDA standards.

- ANSI raised face or Flat face flanged joints should be avoided.

- Ferrules and ferrule connections should be as short as possible to minimize dead legs.

- All Product contact fittings should be self-drainable.

- Nameplates for tagging equipment should be constructed from SS 304 and have minimum crevices.

- Rupture disc should be installed as close as possible to the system’s highest point.

- Sample valves located on vessel shell shall be hygienic, either flush with the inside of the vessel or with a target L/D ratio of 3:1.

- Nozzle connections less than 1 inch should not be used.

- Fittings and other accessories used in the vessel should satisfy the Dimensional tolerances recommended by ASME BPE 2009/07. Refer part DT ASME BPE 2009 for further details on the same.

- Typical Design for ingold Port	 shall be used.

### Sight and Light glass

- The surface of the sight and light glass shall be integral, continuous and free of cracks,    crevices and Pits.

- Cracked glass shall not be used.

- Sufficient illumination bulb with adjustable timer to get off.

### Bottom connections

- Flush bottom valve is in the scope of the Process tank supplier and it has to be considered as a bought out component.

- Flush bottom valve should be from an approved valve supplier like Nova septic and should be manual diaphragm valve.

### Surface finish of the vessel

- The vessel should be mechanically polished and electro polished.

- Ra max should not be more than 0.4µm

- Refer Table SF-7 ASME BPE 2009 for Acceptance criteria for product contact surface finished of vessels.

### Vessel Insulation

| Sl. No | Description | Specification |
| --- | --- | --- |
| 1. | Insulation | Covering the cylindrical and bottom dish portion of the vessel. |
| 2. | Surface Temperature | Ambient (19 Deg. C) |
| 3. | Insulation type | Hot & cold |
| 4. | Insulation material | Aluminum foil, followed by mineral wool (Density: 48 kg/cu.m. Free Cl<25 ppm) Followed by aluminum foil and PUF (Density: 40 kg/cu.m.) Filled between the aluminum foil and the outer cladding sheet (3 mm thick in AISI 304) |
| 5. | Cladding | 3mm AISI 304 – fully welded construction |

NOTE:

- All nozzles on jacket to be provided with PTFE (non-asbestos Teflon wrapped gasket) (Champion AF 120 – 0.5 mm X 6 layers) with counter flange in AISI 304 and bolts and nuts in AISI 304.

- All top dish nozzles to be provided with PTFE gasket with counter flange/ferrule in AISI 316L and bolts and nuts/clamp in AISI 316

- All spare nozzles to be provided with Blind flanges/ferrule of the same MOC as the nozzle.

- Earthing to be provided on the vessel leg

- The clear size of the civil cut-out required for the erection to be indicated by vessel manufacturer.

- The nozzle orientation is only indicative and can change based on the requirements of the process.  All orientations are with respect to the manhole of the top dish.

- Erection & Commissioning shall be in vendor’s scope

- Empty & Dynamic weight of the vessel to be indicated by the vendor

- Vendor has to provide mechanical design calculations and insulation thickness calculations along with offer

# Revision History

| Version Number | Details of revision | Effective Date |
| --- | --- | --- |
| 01 | Newly introduced | December 10, 2015 |

# 8. Approval Details

| DEPARTMENT | NAME | DESIGNATION | SIGNATURE & DATE |
| --- | --- | --- | --- |
| PREPARED BY |  |  |  |
| BG |  |  |  |
| REVIEWED BY |  |  |  |
| BG |  |  |  |
| ED |  |  |  |
| IT |  |  |  |
| EHS |  |  |  |
| Validation Team Head |  |  |  |
| APPROVED BY |  |  |  |
| QA |  |  |  |